This document has been developed from the Phase 1 
document number, Project1-FEED CONTRACTOR-FACILITY-0000-PR-LST-0001 Rev 006, 
incorporating SAP coding requirements and technical 
updates.

PROJECT1 DEVELOPMENT PROJECT

EQUIPMENT LIST

| 002 | Approved for Design | LY" |  | CF | DATE |
| --- | --- | --- | --- | --- | --- |
| 001 | Issued for Approval | JG | JL | CF | DATE |
| 000 | Issued for Review | JG | JL | CF | DATE |
| A01 | Inter-discipline check | JG | JL |  | DATE |
| REV | DESCRIPTION | ORIG | CK'D | APP'D | DATE |

ORIGINAL DRAWING SIZE:	A3

SCALE:	NONE

EQUIPMENT LIST

CLIENT :	COMPANY

ELECTRONIC FILE: Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001

Project1-FEED CONTRACTOR--MUL-E000- PR-LST-0001	SH. 1 of 35

PROJECT1 DEVELOPMENT PROJECT EQUIPMENT LIST

Page 1 of 35 Project1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 Rev 002

Rev.

002

|  |  | EQUIPMENT LIST DETAILS GENERAL NOTES & HOLDS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  |  | Client | COMPANY | Checked By | JL |  |
|  |  | Location | NOTES & HOLDS | Approved By | CF |  |
|  |  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  |  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

General Notes: 1 - Pages have been ordered as per System Code. 2 - Notes specific to equipment are listed in their relevant sheets. 3 - Capacity of FACILITY is 2.8 MSCMD (i.e. 2.0 MSCMD gas sales volume plus 10% shrinkage for LPG recovery at Location O plus 20% swing factor (DCQ) plus 0.1 MSCMD allowance for fuel gas). 4 - Operating conditions taken from Heat and Material balances: PROJECT1-FEED CONTRACTOR-PF01-E000-PR-CAL-0001 Heat and Material Balance: Lean Summer PROJECT1-FEED CONTRACTOR-PF01-E000-PR-CAL-0002 Heat and Material Balance: 30% Lean, 70% Rich Summer PROJECT1-FEED CONTRACTOR-PF01-E000-PR-CAL-0003 Heat and Material Balance: Lean Winter PROJECT1-FEED CONTRACTOR-PF01-E000-PR-CAL-0004 Heat and Material Balance: 30% Lean, 70% Rich Winter 5 - Aquifer water will be supplied to the FACILITY from a local aquifer well drilled at Project1. A back-up aquifer water supply will be available from the Tieret Well via a pipeline. (HOLD 2) 6 - All skids and packages will be manufactured with a requirement of easy accessibility for operation and maintenance with minimal screwed fittings. This shall be reviewed further in EPC. 7 - For more details for sizing basis of some equipment, refer to the relevant technical note: PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0001 Technical Note - FACILITY Turndown Requirements PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0002 Technical Note - Design Pressure For Mobile Units at Wellhead PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0003 Technical Note - Gas Treating Temperature PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0004 Technical Note - Equipment required for Liquid Removal from Gas PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0005 Technical Note - Purge Gas For Flare Header PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0006 Technical Note - Produced Water Disposal PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0007 Technical Note - Mercury Removal Unit Location PROJECT1-FEED CONTRACTOR-FLO-0000-PR-TNO-0001 Technical Note - Hydrate Inhibition PROJECT1-FEED CONTRACTOR-MUL-0000-PR-TNO-0001 Technical Note - Flowline Pressure Protection 8 - The basis for material selection is discussed in the following correspondences: PEL(UK)-RFI-000094 date 20/07/2012 COMPANY-MEMO-000645 date 24/07/2012 9 - The following information are preliminary in the absence of vendor data: Weights, Vendor Package Skid Dimensions, Installed Power for pumps and compressor motors, Column / Vessel Internals, Number of Air Coolers and Heat Exchanger Shells. 10 - The changes in Rev 002 of the Equipment List are based on Design and Cost Improvement Report (PROJECT1-FEED CONTRACTOR-MUL-E000-MD-REP-0004) and subsequently issued Design Change Notices (DCNs). Holds: 1 - Blowdown Study to confirm the minimum design temperatures of equipment (EPC). 2 - Aquifer water source and associated equipment (e.g. wells, pipelines) (EPC). 3 - To be confirmed (EPC). 4 - Package information to be confirmed by Supplier (EPC).

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 2 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 110 
GAS WELLS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | GAS WELLS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 110-PNL-0101 TO 0108 | Wellhead Solar Power System | 8 |  | Notes 2, 6 |  |  | 1.0 |  |  |  |  |  | 10 x 4 x 3 | 1.5 | Vendor Standard |  |  |  |  |  |
| 110-XR-0109 TO 0116 | Chemical Injection Package | 8 |  | Note 2 | 0.12m3/day NOTE5 |  | 0.25 |  | 383 | 82 | -5 / 85 | 50 | 3.0 x 2.0 x 2.5 | 4 each | 316L SS | API 675 |  |  | For Corrosion Inhibitor |  |
| 110-V-0117 | Mobile Test Separator | 1 |  | Notes 1, 4 | Gas: 0.425 MSCMD Condensate: 3300 SBPD Water: 1100 SBPD |  |  |  | 94 | 82 | -25 / 85 | 50 | 5.7 x 2.3 x 2.5 | 15 | CS with 316L clad & 316 internals | ASME VIII Div 1 | Yes |  | Packaged item on rental basis. |  |
| 110-F-0118 | Sand Filter | 1 |  | Note 1 |  |  |  |  | 383 | 280 | -25 / 85 | 50 | 1.0 x 1.0 x 3.0 | 3.0 | CS with 316L clad & 316 internals |  |  |  | 150 micron mesh. |  |
| 110-HP-0119 TO 0126 | Wellhead Control Panel | 8 |  | Note 2 |  |  |  |  |  |  |  |  | 3.0 x 2.0 x 2.0 | 3.0 | Vendor Standard | API 554 |  |  | Includes Hydraulic Well Control Panel (HWCU) |  |
| 110-WHG-0151 TO 0158 | Wellheads | 8 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR-PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not in Contractor's Scope. |  |
| NOTE: - One mobile skid mounted Mobile Test Separator and Sand Filter on rental basis common for all wells (Technical Note - Design Pressure For Mobile Units at Wellhead; PROJECT1-FEED CONTRACTOR-FACILITY-0000-PR-TNO-0002). - Well site Solar Panels, Chemical Injection Package, Hydraulic Well Control Unit (HWCU) are required per well. - Deleted. - Wellhead Mobile Test Separator package includes Test Separator, Ground Flare and Ignitor Package, Choke Manifold and Transfer Pump. - Corrosion / Scale Inhibitor injection rates to be confirmed during EPC Phase. - Wellhead electrical load shall be optimised in order to reduce the power consumption and reduce the size of the Solar Panel during the EPC Phase. - Equipment Tag 110-PK-0127 (Wellhead Water-bath Heater) has been deleted in Rev 002 of this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 3 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 200 CONDENSATE STABILISATION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | CONDENSATE STABILISATION | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 200-HE-1401 | Pre-Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Shell & Tube |  | 1092 (Note 1) |  | 40 | Tube: 25 Shell: 10 | Tube: 19.6 Shell: 5.3 | Tube: -25 / 210 Shell: -25 / 210 | Tube: In= 20 Out= 50 Shell: In= 125, Out= 79.2 | 0.6 ID x 6.5 T/T | 3.6 | Shell: CS • 3mm CA Tubes: 316L SS Note 2, 3 | TEMA R ASME VIII DIV 1 |  | Yes |  |  |
| 200-V-1402 | Water / Condensate Separator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Three Phase Horizontal Separator |  |  |  |  | 25 | 19.1 | -25 / 85 | 50 | 2.3 ID x 5.7 T/T | 18.0 | CS • 3mm CA with Internal Lining and 316L SS Internals Note 5 | ASME VIII Div 1 | Yes |  | Plate pack internals. Weight does not include internals. |  |
| 200-HE-1403 | Condensate/ Condensate Heat Exchanger | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Shell & Tube |  | 1340 (Note 1) |  | 54 | Tube: 25 Shell: 10 | Tube: 18.8 Shell: 6.5 | Tube: -25 / 210 Shell: -25 / 210 | Tube: In= 50 Out= 114 Shell: In= 177, Out= 125 | 0.65 ID x 6.5 T/T | 4.0 | Shell: CS • 3mm CA Tubes: 316L SS Note 2, 3 | TEMA R ASME VIII DIV 1 |  | Yes | Maximum duty and maximum temperature are not necessarily coincident. |  |
| 200-C-1404 | Condensate Stabilisation Column | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Packing |  |  |  |  | 10 | 6.5 | -5 / 210 | 177 | 1.1 ID x 13.3 T/T Note 4 | 7.5 | CS with 316L SS Clad and 316L SS Internals Note 3 | ASME VIII Div 1 | Yes | Yes | Weight does not include internals. |  |
| 200-V-1405 | Water Draw Off Vessel | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Vertical |  |  |  |  | 10 | 6.4 | -5 / 210 | 95 | 0.8 ID x 2.0 T/T | 1.3 | CS • 3mmCA with Internal Lining and 316L SS Internals Note 5 | ASME VIII Div 1 | Yes | Yes |  |  |
| 200-HE-1406 | Condensate Reboiler | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Kettle Type |  | 2791 (Note 1) |  | 170 | Tube: FV / 15 Shell: FV / 10 | 6.5 | Tube: -5 / 235 Shell: -5 / 210 | Tube: 220 Shell: 177 | 0.9 / 2.0 ID x 7.5 L | 8.0 | Shell: CS•316L SS Clad Tube Side: CS - 3mm CA Tubes: 316L SS | TEMA R ASME VIII DIV 1 |  | Yes | Maximum duty and maximum temperature are not necessarily coincident. |  |
| 200-CLR-1407 | Condensate Run Down Cooler | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0002 | Air Cooler |  | 1255 (Note 1) | 44 | 178 (Bare Tube) | 10 | 4.8 | -5 / 135 | In: 115 Out: 60 | 3.7 ID x 7.3 L | 13.5 | Heads: CS • 3mm CA Tubes: CS • Al Fins | API 661 ASME VIII DIV 1 |  |  | 1 bay - 2 fans per bay. Total Fan duty = 27 kW |  |
| NOTE: - This has been pro-rated from the Rich Winter Heat and Material Balance to correspond to 7,000 SBPD condensate product from FACILITY. - Tube sheet / channel materials: CS • 316L SS Clad - Solid CRA may be selected instead of CS Clad where cost / fabrication benefits exist. - Condensate stabilisation column dimensions are based on structured packing internals. - CS • 316L SS Clad is alternate material to internal lining. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 4 of 35

EQUIPMENT LIST DETAILS

SYSTEM CODE: 210

TEST SEPARATION AND METERING

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | TEST SEPARATION AND METERING | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 210-V-0301 | Test Separator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0001 | Three Phase Vertical vessel | Gas: 0.425 MSCMD Condensate: 3283 SBPD Water: 1094 SBPD |  |  |  | FV / 65 | 54 | -25 / 85 | 50 | 1.6 x 6.1 (ID x TT) | 22.0 | CS with 316L SS Clad and 316L SS Internals | ASME VIII Div 1 | Yes |  | The weight of vessel does not include internals. Includes 3m3 slug volume |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 5 of 35

EQUIPMENT LIST DETAILS

SYSTEM CODE: 220

GAS RECEPTION AND SEPARATION

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | GAS RECEPTION AND SEPARATION | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
|  |  |  |  |  | HP Header Gas : 2.83 MSCMD |  |  |  |  | HP: 55.3 |  |  |  |  |  |  |  |  | Skid mounted, 8 slot manifold comprising HP, MP, LP and test production header. |  |
| 220-XX-0401 | Inlet Manifold - HP, MP, LP & Test Header | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- |  | Liquid: 10355 SBPD |  |  |  | 94 | MP: 20.0 | -25 / 85 | 50 | 19 m x 4.5 m | 80 | LTCS + 4.5 mm CA | ASME B31.3 |  |  | HP: 16 x 19 m long; MP: 16 x 19 m long; LP: 16 x | 002 |
|  |  |  | PFD-0001 |  | (Note 1) |  |  |  |  |  |  |  |  | Note 4 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | LP: 7.0 |  |  |  |  |  |  |  |  | 19 m long; Test: 6 x 19 m long |  |
|  |  |  |  |  | Test Header |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 4 |  |
|  |  |  |  |  | 0.425 MSCMD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Shell: 160 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Tube: FV |  | Tube: |  |  |  | Shell: CS + 3mm CA |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Tube: |  | Tube: In: |  |  |  |  |  |  |  |  |
| 220-HE-0402 | Inlet Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- | Shell & Tube |  | 1,903 |  | 44 | / 94 | 54.8 | -25 / 235 | 10°C | 0.7 ID x 6.5 T/T | 5.0 | Tubes: UNS S32750 | TEMA R |  | Yes |  |  |
|  |  |  | PFD-0001 |  |  |  |  |  | Shell: FV |  | Shell: -5 / |  |  |  | Tubesheet: UNS S32750 | ASME VIII DIV 1 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Shell: 10 |  | Out: 25°C |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | / 15 |  | 235 |  |  |  | Channel: UNS S32750 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | (Note 3) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Gas: 2.83 MSCMD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Condensate: 9309 SBPD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 220-V-0403 | Inlet Separator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0001 | Three Phase Vertical vessel | (Note 1) |  |  |  | FV / 65 | 54.3 | -25 / 85 | 50 | 2.2 ID x 7.3 T/T | 42.0 | CS with 316L SS Clad and 316L SS Internals | ASME VIII Div 1 | Yes |  | Weight does not include internals. Includes 3 m3 slug volume. |  |
|  |  |  |  |  | Water: 1046 SBPD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. This has been pro-rated from the Rich Winter Heat and Material Balance to correspond to 7,000 SBPD condensate product from FACILITY. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Inlet manifold will have provision to extend the headers for connection of future wells. - During summer, the Inlet Heater will be bypassed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Skid shall be divided into two units for transportation purposes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 6 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 232 OFFGAS COMPRESSION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | OFFGAS COMPRESSION | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| Offgas Compression Train consists of (Note 1,2): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 232-VG-1001 | 1st Stage Offgas Compression Suction Scrubber | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Vertical Vessel |  |  |  |  | FV / 13 | 5.4 | -25 / 150 | 74.6 | 0.8 ID x 1.9 T/T | 1.5 | CS • 316L SS Clad with 316L SS Internals | ASME VIII DIV 1 | Yes | Yes |  | 002 |
| 232-K-1002 | 1st Stage Offgas Compressor | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Reciprocating (Note 3) | 6336 Sm3/h | 284 | 800 (Note 3) |  | 25 | In: 5.3 Out: 18.5 | -25 / 160 | In: 74.3 Out: 130 |  | 18 (Note 3) | Vendor Specified | API 618 |  |  | 13.2 bar differential. (HOLD 4) | 002 |
| 232-CLR-1003 | 1st Stage Offgas Compression Discharge Cooler | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Air Cooler |  | 1,022 | 30 | 178 (Bare Tube) | FV / 25 | 18.5 | -25 / 160 | In: 130 Out: 60 | 3.7 x 7.3 | 14.0 | 316L SS tubes • header | API 661 ASME VIII DIV 1 |  |  | 1 bay - 2 fans per bay. Total Fan duty = 22 kW | 002 |
| 232-VG-1004 | 2nd Stage Offgas Compression Suction Scrubber | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Vertical Vessel |  |  |  |  | FV / 40 | 18 | -25 / 115 | 47.4 | 1.0 ID x 2.3 T/T | 2.0 | CS • 316L SS Clad with 316L SS Internals | ASME VIII div 1 | Yes | Yes |  | 002 |
| 232-K-1005 | 2nd Stage Offgas Compressor | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Reciprocating (Note 3) | 8158 Sm3/h | 334 | Note 3 |  | 65 | In: 17.9 Out: 55.4 | -25 / 150 | In: 47.4 Out: 120 | Note 3 | Note 3 | Vendor Specified | API 618 |  |  | 37.5 bar differential. | 002 |
| 232-CLR-1006 | 2nd Stage Offgas Compression Discharge Cooler | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Air Cooler |  | 605 | 25 | 89 (Bare Tube) | FV / 65 | 54.7 | -25 / 150 | In: 120 Out: 60 | 1.8 x 7.3 | 6.0 | 316L SS tubes • header | API 661 ASME VIII DIV 1 |  |  | 1 bays - 2 fans per bay. Total Fan duty = 12 kW | 002 |
| 232-P-1007A/B | Compressor Condensate Recycle Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0003 | Positive Displacement | 3.4 m3/h @ 15.0 bar diff. | 1.8 | 3.0 |  | 25 | In: 5.3 Out: 20.6 | -25 / 100 | 74.3 |  | 1.0 | CS | API 676 |  |  |  | 002 |
| NOTE: - Offgas Compressor Train is of 1 x 100% configuration. All equipment for Train B have been deleted in Rev 002 of this document. - Offgas Compressor may be packaged subject to size i.e. compressor (including gear box and driver) and scrubbers on a baseplate with control panel pre-wired and tested. - Two stages on a single shaft. Weight and installed power included in first stage. Weight includes baseplate and motor.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 7 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 240 GAS TREATMENT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | GAS TREATMENT | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 240-FCL-0501 | Gas Treatment Filter Coalescer | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0004 | Vertical Vessel | 2.89 MSCMD |  |  |  | FV /65 | 54.0 | -25 / 85 | 49.5 | 0.74 ID x 5.0 T/T (Note 1) | 13.7 | 316L SS with 316L SS internals (Note 2) | ASME VIII Div 1 | Yes |  | Note 4 |  |
| NOTE: - Vessel dimensions based on vendor information from Phase 1. To be confirmed with supplier during EPC. - CS Clad material may be used in lieu of solid CRA material. - Equipment Tag 240-DR-0502 (Mercury Removal Bed) and 240-FC-0503 (After-Filter for Mercury Removal Unit) have been deferred for procurement until further information regarding mercury content is fully assessed. Accordingly, these items have been deleted in the Rev 002 of this document. - HAZOP recommendation for a standby Filter Coalescer was not considered and 'Management of Change' to be issued by Company. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 8 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 250 GAS CONDITIONING (DEHYDRATION AND DEW POINTING) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | GAS CONDITIONING (DEHYDRATION AND DEW POINTING) | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 250-HE-0701 | Export Gas/ Sweet Gas Heat Exchanger | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0005 | Hair Pin |  | 2,024 |  | 856 (Note 2) | FV / 65 | Tube: 52.0 Shell: 48.7 | -46 / 85 | Tube: In= 55, Out= 30.0; Shell: In= 20.3, Out= 50 | 12 x 1.5 x 2.0 (Note 2) | 30 | Shell: LTCS • 3 mm CA Tubes: 316L SS Tubesheet: 316L SS Channel: 316L SS | ASME VIII Div 1 / TEMA R |  | Yes | Combine with 250-HE-0706 on one skid. |  |
| 250-FCL-0703 | TEG Contactor Filter Coalescer | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0005 | Vertical Vessel | 2.86 MSCMD |  |  |  | FV / 65 | 51.4 | -25 / 85 | 30 | 1.8 ID x 5.3 T/T (Note 1) | 25.0 | CS with 316L Clad and 316L SS Internals | ASME VIII Div 1 | Yes | Yes | Note 1 | 002 |
| 250-C-0704 | TEG Contactor | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0005 | Structured Packing (Note 3, 4) |  |  |  |  | FV / 65 | 51.1 | -25 / 85 (Note 4) | 30 | 1.35 ID x 8.5 T/T (Note 1) | 20.0 | Lower: CS • 316L SS Clad, Upper: CS • 3mm CA 316L SS Internals | ASME VIII Div 1 | Yes | Yes | The weight of column does not include internals |  |
| 250-PK-0705 | TEG Regeneration Package (comprising): | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-PFD-0005 | Note 3 |  |  |  |  |  |  |  |  | 10.5 x 2.6 x 4.8 | 26 | Vendor specified |  |  |  | HOLD 4 |  |
| 250-CLR-0709 | Lean Glycol Air Cooler | 1 |  | Air Cooler (Note 3) |  | 40 | 11 | 20 (Bare Tube) | 65 | 52 | -5 / 100 | In: 86 Out: 60 |  |  | 316L SS Header : Al finned tubes | API RP661 & ASME VIII DIV I |  |  | Part of TEG Regeneration Package 1 bay - 2 fans per bay. Total Fan duty = 5 kW Not installed on skid. |  |
| 250-CND-0710 | Glycol Regeneration Condenser | 1 |  | Reflux Coil (Note 3) |  | 4 |  |  | 3.5 | 0.1 | -5 / 135 | 100 |  |  | 316L SS | API RP661 & ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-HE-0711 | Lean / Rich Glycol Heat Exchanger | 1 |  | Brown fin tube or plate & frame exchanger (Note 3) |  | 200 |  | 16 | 10 (shell), 10 (tube) | 0.2 (shell), 3.5 (tube) | -5 / 235 (tube), -5 / 235 (shell) | Tube: In:=35 Out = 150 Shell: In= 204 Out = 86 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-F-0712A/B | Glycol Particle Filter | 2 |  | Cartridges, vertical, polypropylene elements (Note 3) | 2,250 kg/h |  |  |  | 10 | 3.5 | -5 / 100 | 35 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-F-0713 | Glycol Charcoal Filter | 1 |  | Charcoal, carbon elements (Note 3) | 2,250 kg/h |  |  |  | 10 | 4 | -5 / 100 | 35 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-P-0714A/B | Lean Glycol Pump | 2 |  | Fixed Stroke Reciprocating (Note 3) | 2.0 m3/h | 5 | 7.5 |  | 65 | 53 | -5 / 140 | 86 |  |  | CS | API STD 674 |  | Yes | Part of TEG Regeneration Package. |  |
| 250-C-0715 | Regeneration Column | 1 |  | Random Packing (Note 3) | 2,390 kg/h |  |  |  | 3.5 | 0.3 | -5 / 235 | Bottom: 204 Top: 100.1 |  |  | 316L SS | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-V-0716 | Glycol Flash Drum | 1 |  | Horizontal three phase separator (Note 3) |  |  |  |  | 10 | 4.2 | -5 / 100 | 35.7 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-V-0717 | Glycol Surge Drum | 1 |  | Horizontal vessel (Note 3) |  |  |  |  | 3.5 | 0.3 | -5 / 235 | 204 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. |  |
| 250-HE-0718 | Glycol Reboiler | 1 |  | Electric Heater in Horizontal Vessel (Note 3) |  | 145 | 180 |  | 3.5 | 0.3 | -5 / 235 | 204 |  |  | CS • 3 mm CA | ASME VIII DIV I |  | Yes | Part of TEG Regeneration Package. Additional 20% duty for start up case. |  |
| 250-HE-0706 | Export Gas/ Dry Gas Heat Exchanger | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0006 | Hair Pin |  | 1039 |  | 283 | FV / 65 | Tube: 50.3 Shell: 49.2 | -46 / 85 | Tube: In= 31, Out= 17.7; Shell: In= 5.7, Out= 20.3 | 8.0 x 1.0 x 2.0 | 17 | Shell: LTCS • 3 mm CA Tubes: LTCS Tubesheet: LTCS • 3mm CA Channel: LTCS • 3mm CA | TEMA R ASME VIII DIV 1 |  | Yes | Combine with 250-HE-0701 on one skid. |  |
| 250-HE-0707 | Dry Gas Chiller | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0006 | Shell & Tube Exchanger |  | 1071 |  | 176 | Tube: FV / 65 Shell: FV / 18 | Tube: 49.8 Shell: 3.7 | -46 / 85 | Tube: In= 17.7, Out= 5.8; Shell: In= 0.0, Out= 1.5 | 0.8 ID x 8 T/T | 6.5 | Shell: LTCS • 3 mm CA Tubes: LTCS Tubesheet: LTCS • 3mm CA Channel: LTCS • 3mm CA | TEMA R ASME VIII DIV 1 |  | Yes |  |  |
| 250-V-0708 | Cold Separator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0006 | Vertical Vessel | 2.8 MSCMD |  |  |  | FV / 65 | 49.3 | -46 / 85 | 5.8 | 1.7 ID x 3.2 T/T | 15.0 | LTCS • 3 mm CA with 316L SS Internals | ASME VIII DIV 1 | Yes | Yes |  |  |
| NOTE: - The TEG Contactor Filter Coalescer (250-FCL-0703) includes a 3-phase knock-out section at the bottom to separate water / amine and condensate streams from the cooled gas. Vessel dimensions to be confirmed with Supplier during EPC. Accordingly, Equipment Tag 250-KO-0702 (Sweet Gas KO Drum) has been deleted in Rev. 002 of this document. - Exchanger type, area and dimensions (pro-rated for revised duty) are based on vendor information from phase 1. - Package equipment and duties are typical. To be confirmed with supplier during EPC. - TEG Contactor will be supplied by the Regeneration Package supplier. Design temperature to be confirmed with supplier for failure of Lean Glycol Cooler. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 9 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 270

GAS EXPORT AND METERING

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | GAS EXPORT AND METERING | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 270-XX-1202 | Export Gas Fiscal Metering Package | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0009 | Includes On-line Gas Chromatograph and dew-point analyser | 2.7 MSCMD |  |  |  | 94 | 48.3 | -25 / 85 | 50.2 | 9.0 (L) x 4.0 (W) | 20.0 | CS (Vendor Specified) | AGA 9 |  |  |  |  |
| NOTE: 1. Metering package details will be confirmed during EPC Phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 10 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 280 GAS SWEETENING (AMINE) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | GAS SWEETENING (AMINE) | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 280-C-0601 | Amine Absorber Column | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0004 | Tray (HOLD 3) (Note 1, 2) | 1.2 MSCMD |  |  |  | FV / 65 | 54 | -25 / 85 (Note 2) | 68.9 | 1.3 ID x 20 T/T | 35 | CS with 316L SS Clad and 316L SS Internals | ASME VIII Div 1 | Yes | Yes | The weight of column does not include internals. |  |
| 280-PK-0602 | Amine Regeneration Package (comprising): | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0004 | Note 1, 3 |  |  |  |  |  |  |  |  | Two Skids: 1 - 14.0 (L) x 4.0 (W) 2 - 11.5 (L) x 2.5 (W) | 35 |  |  |  |  | HOLD 4 |  |
| 280-V-0603 | Amine Regeneration Flash Drum | 1 |  | (Note 1) | 33 m3/h |  |  |  | 15.0 | 9.0 | -5 / 85 | 60 | Included in Package Skid |  | CS with 316L SS Clad | ASME VIII Div 1 |  | Yes | Part of Amine Regeneration Package |  |
| 280-HE-0604 | Amine Regeneration Lean / Rich Exchanger | 1 |  | Plate & Frame Type Exchanger (Note 1) |  | 1,020 |  | 2,500 | 15.0 | Rich: 9.0 Lean: 3.5 | -5 / 150 | Rich: 60 - 90 Lean: 110 - 82 | Included in Package Skid |  | 316L SS | ASME VIII DIV I | Yes | Yes | Part of Amine Regeneration Package |  |
| 280-C-0605 | Amine Regeneration Stripper Column | 1 |  | HOLD 2 (Note 1) | 33 m3/h |  |  |  | 5.0 | 1.0 | -5 / 150 | 110 | 1.3 x 20.0 (ID x TT) (Free Standing) | 11 | CS with 316L SS Clad | ASME VIII DIV I | Yes | Yes | Part of Amine Regeneration Package |  |
| 280-CLR-0606 | Amine Regeneration Condenser | 1 |  | Air Cooler (Note 1) |  | 100 | 8 | 30 (Bare Tube) | 5.0 | 0.9 | -5 / 150 | In: 80 Out: 60 | 5.0 x 2.4 x 3.9 | 5 | Duplex SS Header, 316 Tubes | API 661 & ASME VIII DIV I | Yes | Yes | Part of Amine Regeneration Package 1 bay - 2 fans per bay. Total Fan duty = 5.5 kW Induced draft type assumed. |  |
| 280-V-0607 | Amine Regeneration Condensate Drum | 1 |  | (Note 1) |  |  |  |  | 5.0 | 0.7 | -5 / 150 | 60 | Included in Package Skid |  | 316L SS | ASME VIII DIV I |  |  | Part of Amine Regeneration Package |  |
| 280-P-0608A/B | Amine Regeneration Reflux Pump | 2 |  | Positive Displacement (Note 1) | 2.0 m3/h | 0.8 | 1.5 |  | 5.0 | 0.8 | -5 / 150 | 60 | Included in Package Skid |  | 316L SS | API 676 |  |  | Part of Amine Regeneration Package |  |
| 280-P-0609A/B | Amine Regeneration Lean Solution Booster Pump | 2 |  | Centrifugal (Note 1) | 33 m3/h | 11 | 15 |  | 5.0 | 3.5 | -5 / 150 | 110 | Included in Package Skid |  | CS / 316L SS Impeller | API 610 |  | Yes | Part of Amine Regeneration Package |  |
| 280-CLR-0610 | Amine Regeneration Lean Solution Cooler | 1 |  | Air Cooler (Note 1) |  | 760 | 22 | 85 (Bare Tube) | 5 | 3.5 | -5 / 150 | In: 82 Out: 60 | 3.0 x 9.0 x 4.0 | 11 | 316L SS | API 661 ASME VIII DIV 1 | Yes |  | Part of Amine Regeneration Package 2 bays - 2 fans per bay. Total Fan duty = 13 kW Induced draft type assumed. |  |
| 280-F-0611A/B | Mechanical Filter | 2 |  | (Note 1) | 33 m3/h |  |  |  | 15.0 | 9.0 | -5 / 85 | 60 | Included in Package Skid |  | CS | ASME VIII DIV I | Yes |  | Part of Amine Regeneration Package |  |
| 280-F-0612A/B | Carbon Filter | 2 |  | (Note 1) | 6 m3/h |  |  |  | 5 | 3.5 | -5 / 150 | 60 | Included in Package Skid |  | CS | ASME VIII DIV I | Yes |  | Part of Amine Regeneration Package |  |
| 280-F-0613A/B | Mechanical Filter | 2 |  | (Note 1) | 6 m3/h |  |  |  | 5 | 3.5 | -5 / 150 | 60 | Included in Package Skid |  | CS | ASME VIII DIV I | Yes |  | Part of Amine Regeneration Package |  |
| 280-P-0614A/B | Amine Regeneration Lean Solution Pump | 2 |  | Centrifugal (Note 1) | 33 m3/h | 65 | 90 |  | 65 | 56.0 | -5 / 85 | 60 | Included in Package Skid |  | CS / 316L SS Impeller | API 610 |  |  | Part of Amine Regeneration Package |  |
| 280-HE-0615 | Amine Regeneration Reboiler | 1 |  | Shell & tube (kettle) (Note 1) |  | 1,500 |  | 160 | 5.0 | 1.2 | -5 / 150 | 110 | Included in Package Skid |  | CS + 3mm CA 316L SS Tubes | TEMA R ASME VIII DIV 1 |  | Yes | Part of Amine Regeneration Package. Heating Medium supply to reboiler. |  |
| NOTE: - Package equipment and duties are typical. To be confirmed with supplier during EPC. - Amine Absorber Column will be supplied by the Amine Regeneration package supplier. Design temperature to be confirmed with supplier for failure of lean amine cooler. - Automated sampling and analysis of amine solution to be provided utilising mass spectrometry type analytical system. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 11 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 310

CONDENSATE EXPORT LINE

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | CONDENSATE EXPORT LINE | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 310-PLR-1601 | Export Condensate Pig Launcher | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0010 |  |  |  |  |  | 94 | Current: 41 Future: 82 (Note 3) | -25 / 85 | 60 | Diameter (Minor/Major Barrel): 6"/10.75" NB Length: 5.8 m | 1.00 | CS • 3mm CA | ASME B31.4/ ASME VIII Div 1 |  |  |  |  |
| 310-PL-1602 | Export Condensate Pipeline | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0010 |  | 10,000 SBPD (Note 1) |  |  |  | 94 | Current: 41 Future: 82 (Note 3) | -25 / 85 | 60 | 11.7 km & 6" NB (7.11 mm thickness) | 340 | CS API 5L X52• 3 mm CA Externally coated. | ASME B31.4 / NT 109.02 |  |  |  |  |
| 310-PLR-1603 | Export Condensate Pig Receiver | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0010 | Note 2 |  |  |  |  | 94 | 72 (Note 3) | -25 / 85 | 46 | Diameter (Minor/Major Barrel): 6"/10.75" NB Length: 8.5m | 1.00 | CS • 3mm CA | ASME B31.4/ ASME VIII Div 1 |  |  |  |  |
| 310-PIT-1604 | Export Condensate Drain Sump | 1 |  | Note 2 |  |  |  |  | ATM | ATM | -5 / 85 | AMB | 1.0 x 1.0 x 1.0 |  | Lined Concrete |  |  |  |  |  |
| 310-PNL-1605 | PROJECT Z Solar Power System | 1 |  | Note 2, 4 |  |  | 0.4 |  |  |  |  |  | 5.0 x 4.0 x 3.0 | 0.8 | Vendor Standard | API 554 |  |  |  |  |
| NOTE: - The Export Condensate Pipeline is designed for 10,000 SBPD; the design margin in the capacity enables to bring down the storage inventory after a PROJECT Z shutdown. - Equipment located near tie-in with PROJECT Z facilities. - Operating pressure depends on throughput of PROJECT Z pipeline. Currently PROJECT Z is operating a relatively low capacity and export pipeline operating pressure is consequently lower. - PROJECT Z electrical load shall be optimised in order to reduce the power consumption and reduce the size of the Solar Panel during the EPC Phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 12 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 320 
GAS EXPORT LINE

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | GAS EXPORT LINE | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY |  | Package Notes | Capacity | Duty kW | Instal. Power kW | PFD No.(each) Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 320-PLR-1101 | Export Gas Pig Launcher | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0009 |  |  |  |  |  | 94 | 46.4 | -25 / 85 | 49.4 | Diameter (Minor/Major Barrel): 24' /30' NB Length: 7.2m | 2.70 | CS + 3mm CA | ASME B31.8/ ASME VIII Div 1 |  |  |  |  |
| 320-PL-1102 | Export Gas Pipeline | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0009 |  | 2.7 MSCMD |  |  |  | 94 | 46.4 | -25 / 85 | 49.4 | 51.8 km x 24' (9.53mm thickness) | 7310 | CS API 5L X65 + 1.5 mm CA | ASME B31.8 |  |  | Future injection pressure at Location Z Receiving Station is 70barg. External 3 layer PE coating (3mm thick). |  |
| 320-XX-1103 | Export Gas Pipeline Block Valve Station | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR-PFD-0009 |  |  |  |  |  | 94 | 46.4 | -25 / 85 | 49.4 |  | 4.5 | Vendor Standard | ASME B31.8 |  |  | 24' NB Block Valve. |  |
| 320-PLR-1104 | Export Gas Pig Receiver | 1 | PROJECT1-FEED CONTRACTOR-OF02-E000-PR- PFD-0009 |  |  |  |  |  | 94 | 46.4 | -25 / 85 | 49.4 | Diameter (Minor/Major Barrel): 24' /30' NB Length: 10.6m | 2.7 | CS + 3mm CA | ASME B31.8/ ASME VIII Div 1 |  |  |  |  |
| 320-PNL-1105 | Block Valve Station Solar Power System | 1 |  | Note 1 |  |  | 0.4 |  |  |  |  |  | 5.0 x 4.0 x 3.0 | 0.8 | Vendor Standard | API 554 |  |  |  |  |
| 320-PIT-1106 | Export Gas Drain Sump | 1 |  |  |  |  |  |  | ATM | ATM | -5 / 85 | AMB | 1.0 x 1.0 x 1.0 |  | Lined Concrete |  |  |  |  |  |
| 320-PNL-1107 | HRS Solar Power System | 1 |  | Note 1 |  |  | 0.4 |  |  |  |  |  | 5.0 x 4.0 x 3.0 | 0.8 | Vendor Standard | API 554 |  |  |  |  |
| NOTE: 1. HRS and Block Valve Station electrical loads shall be optimised in order to reduce the power consumption and reduce the size of the Solar Panel during the EPC Phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 13 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 330

CONDENSATE STORAGE AND METERING

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | CONDENSATE STORAGE AND METERING | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS + 3 mm CA |  |  |  |  |  |
| 330-TK-1501 | Condensate Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0008 |  | 1600 m3 (Note 3, 5) |  |  |  | 0.17 | ATM | -5 / 85 | 60 | 15.0 ID x 12.6 H | 100 | with non-metallic internal lining | API 650 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS + 3 mm CA |  |  |  |  |  |
| 330-TK-1502 | Condensate Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0008 |  | 1600 m3 (Note 3, 5) |  |  |  | 0.17 | ATM | -5 / 85 | 60 | 15.0 ID x 12.6 H | 100 | with non-metallic internal lining | API 650 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Case: Cast Iron |  |  |  | Note 1 |  |
|  |  |  |  | Vertical Multistage | 33 m3/h @ 40.3 bar |  |  |  |  |  |  |  |  |  | Impeller: Cast |  |  |  |  |  |
| 330-P-1504A/B | Condensate Export Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0008 | Can Type Centrifugal Pump | dp (Note 1) | 62 | 75 |  | 94 | 40 | -5 / 85 | 63 |  | 3.4 each | Iron API 610 Material Designation I-1 | API 610 |  |  | 2 x 50% pumps. In future, the pumps may operate with a dp of 82 bar and may require revamping or replacement. | 002 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Case: Cast Iron |  |  |  |  |  |
|  |  |  |  | Vertical Multistage |  |  |  |  |  |  |  |  |  |  | Impeller: Cast |  |  |  |  |  |
| 330-P-1505A/B | Condensate Recycle Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0008 | Can Type Centrifugal Pump | 16.6 m3/h @ 20.5 bar dp | 21 | 30 |  | 28 | 20.5 | -5 / 85 | 60.8 |  | 1.7 | Iron API 610 Material | API 610 |  |  | Electric Driven. 2x100% pumps. | 002 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Designation I-1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS + 3 mm CA |  |  |  |  |  |
| 330-XX-1507 | Condensate Metering Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0008 |  | 66 m3/h (Note 1) |  |  |  | 94 | 82.1 | -5 / 85 | 63 |  | 2.0 | (Vendor specified) |  |  |  | LACT unit. (Note 6) |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Each Condensate Export Pump capacity corresponds to a nominal production rate of 5000 SBPD. Both export pumps may need to be in operation to bring down the storage inventory after a PROJECT Z shutdown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - Condensate Recycle Pump is designed for turndown and reprocessing requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - Condensate storage tanks: Each tank has 2 days storage capacity corresponding to a nominal production rate of 5000 SBPD. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Equipment items 330-TK-1503 (Offspec Condensate Storage Tank), 330-P-1506 (Offspec Condensate Recycle Pump) and 330-P-1504C (Future Condensate Export Pump) have been deleted in Rev. 002 of this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - Condensate storage tanks: Specified capacity is the net required storage capacity. Tank dimensions include margins for ullage and dead volume and are based on API 650 typical size. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - LACT Unit comprises coriolis mass flow meter, with RVP analyser, BS&W monitor and density measurement. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 14 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 360 
WELL FLOWLINES

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | WELL FLOWLINES | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Diameter (minor/major |  |  | ASME VIII DIV 1 / |  |  |  |  |
| 360-PLR-0201 | Mobile Pig Launcher | 1 |  | Note 1 |  |  |  |  | FV / 94 |  | -25 / 85 |  | barrel): 6 /10.75 NB | 2.0 | CS + 3mm CA |  |  |  | Packaged item |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B31.8 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Length = 5.8m |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Diameter (minor/major |  |  | ASME VIII DIV 1 / |  |  |  |  |
| 360-PLR-0202 | Mobile Pig Receiver | 1 |  | Note 1 |  |  |  |  | FV / 94 |  | -25 / 85 |  | barrel): 6 /10.75 NB | 2.0 | CS + 3mm CA |  |  |  | Packaged item |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B31.8 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Length = 8.5m |  |  |  |  |  |  |  |
| 360-PL-0210 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 61 | -25 / 85 | 50 | 7.5 km & 6 NB | 220 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 2-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0211 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 59 | -25 / 85 | 50 | 7.4 km & 6 NB | 220 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 2-2. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0212 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 62 | -25 / 85 | 50 | 6.3 km & 6 NB | 185 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 5-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0213 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 68 | -25 / 85 | 50 | 20.0 km & 6 NB | 580 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 6-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0214 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 82 | -25 / 85 | 50 | 20.5 km & 6 NB | 590 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 4-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0215 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 73 | -25 / 85 | 50 | 13.0 km & 6 NB | 380 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 3-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0216 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 59 | -25 / 85 | 50 | 3.4 km & 6 NB | 120 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Well 1-1. Notes 4, 7 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0217 | Wellhead flowline | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 94 | 58 | -25 / 85 | 50 | 1.5 km & 6 NB | 50 | CS API 5L X52 + 3mm CA | ASME B31.8 / |  |  | Project1-1. Notes 4, 7 | 002 |
|  |  |  | PFD-0011 |  |  |  |  |  |  |  |  |  |  |  |  | NT109.01 |  |  |  |  |
| 360-PL-0218 | Methanol Line for Project1 | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 383 | Approx. | -5 / 85 | 50 | 1.5km & 2 NB | 17 | CS API 5L X52 + 3mm CA | ASME B36.10 |  |  | Note 5, 8 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  | 280 |  |  |  |  |  | B31.4/ NT 109.02 |  |  |  |  |
| 360-PL-0219 | Methanol Line for Well 2 1/2 | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 383 | Approx. | -5 / 85 | 50 | 10.54km & 2 NB | 120 | CS API 5L X52 + 3mm CA | ASME B36.10 |  |  | Note 5, 8 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  | 280 |  |  |  |  |  | B31.4/ NT 109.02 |  |  |  |  |
| 360-PL-0220 | Methanol Line for Well 4 / Well 3 / Well 5 | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 383 | Approx. | -5 / 85 | 50 | 24.19km & 2 NB | 270 | CS API 5L X52 + 3mm CA | ASME B36.10 |  |  | Note 5, 8 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  | 280 |  |  |  |  |  | B31.4/ NT 109.02 |  |  |  |  |
| 360-PL-0221 | Methanol Line for Well 6 / Well 1 | 1 | PROJECT1-FEED CONTRACTOR-OF01-E000-PR- |  |  |  |  |  | 383 | Approx. | -5 / 85 | 50 | 19.75km & 2 NB | 220 | CS API 5L X52 + 3mm CA | ASME B36.10 |  |  | Note 5, 8 |  |
|  |  |  | PFD-0011 |  |  |  |  |  |  | 280 |  |  |  |  |  | B31.4/ NT 109.02 |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - One portable skid mounted Mobile pig launcher and Mobile pig receiver common for all wells. Dimensions based on typical supplier information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Deleted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - All flowlines to be buried and provided with an external 3 layer PE coating (3 mm thk). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Flowline wall thickness is 7.11mm for all flowlines. Corrosion allowance for flowline specified at 3mm by COMPANY Laboratory in Country2 (FEED Contractor recommendation is for 6mm CA). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - Methanol lines from FACILITY to Wellheads are routed along the flowlines in trenches. Methanol line thickness: 8.74mm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - All Wellheads shall be provided with PSVs including vent stacks. This is for over-pressure protection of the flowlines (Technical Note - Flowline Pressure Protection; PROJECT1-FEED CONTRACTOR-MUL-0000-PR-TNO-0001). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |
| - Pipeline depressuring rate shall confirm the minimum design temperature of the pipelines. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The methanol injection line to the wellheads are represented by lines 2 -XA-360-GGG-25CH3M on P&ID PROJECT1-FEED CONTRACTOR-OF01-E360-PR-PID-0001. See also notes 2, 3 and 4 on the same P&ID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Equipment tags 360-KO-0203 to 0209 (Flowline Relief Knock Out Drums) have been deleted in Rev. 002 of this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 15 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 400 
COOLING MEDIUM

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | COOLING MEDIUM | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 400-PK-1901 | Refrigeration Package (comprising): | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0009 | Note 1, 2 |  | 1071 (thermal) |  |  |  |  |  |  |  |  |  |  |  |  | HOLD 4 |  |
| 400-RCV-1903 | Refrigerant Receiver Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0009 |  | 5.3 m |  |  |  | HOLD 4 | 20.3 | HOLD 4 | 60 | 1.5 ID x 4.5 T/T | 7.0 | Vendor Specified | ASME VIII Div 1 |  | Yes | Part of Refrigeration Package. |  |
| 400-KO-1904 | Suction Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0009 |  |  |  |  |  | HOLD 4 | 3.5 | HOLD 4 | 1.5 | 1.3 D x 2.6 T/T | 3.5 | Vendor Specified | ASME VIII Div 1 |  | Yes | Part of Refrigeration Package. |  |
| 400-CLR-1905 | Condenser (Air Cooler) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0009 | Air Cooler |  | 1,604 | 60 |  | HOLD 4 | 21.0 | HOLD 4 | In: 81 Out: 60 | 12.5 x 4.8 x 5.0 | 20.0 | Vendor Specified | ASME VIII DIV 1 API 661 |  |  | Part of Refrigeration Package. 1 Bay - 2 Fans. Fan duty = 41 kW |  |
| 400-K-1906 | Refrigerant Compressor | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0009 | Screw |  | 532 | 640 |  | HOLD 4 | In: 3.4 Out: 21 | HOLD 4 | In: 1.1 Out: 81 |  | 20.0 | Vendor Specified | API 619 |  | Yes | Note 4 Part of Refrigeration Package. 1 x 100% compressor. | 002 |
| 400-PK-1902 | Refrigeration Storage Vessel Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0009 | Note 3 | 7.5 m |  |  |  | 18 |  | -25 / 85 |  | 1.37 OD x 4.7 T/T | 2.0 | Vendor Specified | ASME VIII Div 1 |  |  | HOLD 4 |  |
| NOTE: - Package equipment and duties are based on vendor information in Phase 1. To be confirmed with supplier in EPC. - Refrigeration based on propane refrigerant. - Tote tank containing refrigerant will be delivered to Site and held in-place on a permanent basis to replenish the system inventory in case of losses. Propane inventory required is approximately 3.5 te. Dimension shown above is for typical 2000 US gallon propane tote tank. - Space and Tie-ins shall be provided within the package unit for future installation of a spare compressor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 16 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 410 
HEATING MEDIUM

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | HEATING MEDIUM | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 410-PK-1801 | Heating Medium Package (comprising): | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0008 | Notes 1, 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HOLD 4 |  |
| 410-EX-1802 | Heating Medium Expansion Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0008 | Note 1 | 4 m3 |  |  |  | 15 | 3.5 | -5 / 235 | 135 | 1.3 ID x 3.9 T/T | 1.0 | Vendor Specified | ASME VIII Div 1 |  | Yes | Part of Heating Medium Package |  |
| 410-P-1803A/B | Heating Medium Circulation Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0008 | Centrifugal (Note 1) | 263 m3/h each @ 7 bar dp | 85 each | 115 |  | 15 | 10 | -5 / 235 | 135 |  | 2.0 | Vendor Specified | API 610 |  | Yes | Part of Heating Medium Package |  |
| 410-F-1804 | Heating Medium Filter | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0008 | Note 1 |  |  |  |  | 15 | 10 | -5 / 235 | 135 |  | 0.5 | Vendor Specified | ASME VIII DIV 1 |  | Yes | Part of Heating Medium Package |  |
| 410-FH-1805 | Heating Medium Fired Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0008 | Note 1 |  | 6,100 |  |  | 15 | 10 | -5 / 235 | 220 | 5.0 OD x 14 H | 27.0 | Vendor Specified | API 560 / 530 |  | Yes | Part of Heating Medium Package |  |
| 410-CLR-1806 | Heating Medium Dump Cooler | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0008 | Air Cooler (Note 1) |  | 2,133 | 37 | 142 (Bare) | 15 | 8.5 | -5 / 235 | 185 | 2.5 x 4.6 | 7.3 | Vendor Specified | API 661 |  |  | Part of Heating Medium Package 2 bays - 2 fans per bay. Total Fan duty = 18 kW |  |
| 410-V-1807 | Heating Medium Drain Vessel | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0008 |  | 35 m3 |  |  |  | 3.5 | 0.05 | -5 / 185 | 135 | 2.6 ID x 7.5 T/T | 8.1 | CS + 3mm CA Externally Coated | ASME VIII Div 1 |  |  |  |  |
| 410-P-1808A/B | Heating Medium Drain Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0008 |  | 5 m3/h @ 8 bar dp | 2.8 | 4.0 |  | 13 | 8.5 | -5 / 185 | 135 |  | 1.0 | CS case with CS impeller. API 610 material designation S-9 | API 610 |  |  |  |  |
| NOTE: - Package equipment and duties are typical. To be confirmed with supplier during EPC. The Heating Medium Package shall comprise several skids. - The heating medium system comprises two temperature levels of hot oil supply to consumers: Heating medium temperature to Condensate Stabilisation Reboiler and Inlet Heater will be 220°C; while that to Amine Regeneration Reboiler will be 160°C. Typical controls within the package unit are shown in the UFD, which should be confirmed by supplier in EPC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 17 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 420 
CHEMICAL INJECTION

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | CHEMICAL INJECTION | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 420-PK-2801 | Chemical Injection Package (Process) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- |  |  | Approx. | Approx. |  | 65 | 55.0 | -5 / 85 | 50 | 3.0 x 4.0 x 3.0 | 16.0 | SS (Vendor | API 675 |  |  | Notes 1, 2, 5 |  |
|  |  |  | UFD-0006 |  |  | 1 | 2 |  |  |  |  |  |  |  | Specified) |  |  |  |  |  |
| 420-PK-2802 | Chemical Injection Package (Produced Water) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- |  |  | Approx. | Approx. |  | 10 | 3.0 | -5 / 85 | 50 | 3.0 x 3.0 x 2.0 | 10.0 | SS (Vendor | API 675 |  |  | Notes 3, 4, 5 |  |
|  |  |  | UFD-0006 |  |  | 1 | 2 |  |  |  |  |  |  |  | Specified) |  |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. The injection chemicals for process typically are corrosion inhibitor, anti-foam, scale inhibitor, demulsifier etc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. The Chemical Injection Package for process comprises of (HOLD 4): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Multicompartmented tank (4 compartments) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Dosing pumps - Multihead dosing pumps (2 x 100%) with 2 heads each are required for Antifoam injection; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Multihead dosing pumps (3 x 50% - 2 operating + 1 standby) with 4 heads each are required for Corrosion Inhibitor. The same applies to Scale Inhibitor injection, if required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 2 x 100% dosing pumps for Demulsifier - Mixers (one per compartment) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. The injection chemicals for produced water are corrosion inhibitor, reverse demulsifier, biocide, etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. The Chemical Injection Package for Produced Water system comprises of (HOLD 4): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Multicompartmented tank (3 compartments) - Dosing pumps (3 operating + 3 standby) - Mixers (3; one per compartment) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. For details, refer Chemical Injection Packages datasheet (PROJECT1-FEED CONTRACTOR-PF01-E420-PR-DAT-0028) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 18 of 35

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | FLARE | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

EQUIPMENT LIST DETAILS SYSTEM CODE: 430

FLARE

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 430-VD-1701 | HP Flare KO Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 |  | Gas: 104427 kg/h (2.8 MSCMD) Liquid: Liquid: 45352 kg/h (57 m3/h) |  |  |  | FV / 10 | 4.0 | -46 / 160 | 50 | 2.8 ID x 6.5 T/T | 12.0 | CS • 3mm CA with non-metallic internal lining | ASME VIII Div 1 |  | Yes |  |  |
| 430-P-1702A/B | HP Flare KO Drum Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 | Centrifugal | 24.2 m3/h @ 2.5 bar dp | 2.9 | 5.5 |  | 15 | 2.9 | -5 / 85 | 50 |  | 1.0 | Case: CS Impeller: CS API 610 Material Designation S-5 | API 610 |  |  |  |  |
| 430-SK-1703 | HP Flare Stack | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 | Package Unit (Note 2) | Gas: 104427 kg/h (2.8 MSCMD) (Note 1) |  |  |  | 10 | 3.2 | -46 / 160 | 50 | Flare stack diameter 18"; Flare tip diameter 16"; Stack ht - 35m (Note 1) | 16.0 | Vendor Specified | API 537 / API 521 |  |  |  |  |
| 430-BIG-1704 | Ignition Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 | Package Unit (Note 3) |  |  |  |  |  |  |  |  |  | 0.7 | Vendor Specified |  |  |  | Common for HP and LP Flare |  |
| 430-VD-1705 | LP Flare KO Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0004 |  | Gas: 32030 kg/h (0.34 MSCMD) Liquid: 9354 kg/h (9.4 m3/h) |  |  |  | FV / 3.5 | 0.3 | -5 / 210 | 50 | 1.8 ID x 5.0 T/T | 4.0 | CS • 3mm CA with non-metallic internal lining | ASME VIII Div 1 |  |  |  |  |
| 430-P-1706A/B | LP Flare KO Drum Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0004 | Centrifugal | 5.5 m3/h @ 2.7 bar dp | 1.3 | 2.2 |  | 8 | 3.2 | -5 / 85 | 50 |  | 1.0 | Case: CS Impeller: CS API 610 Material Designation S-5 | API 610 |  |  |  |  |
| 430-SK-1707 | LP Flare Stack | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 | Package Unit (Note 2) | Gas: 32030 kg/h (0.34 MSCMD) (Note 1) |  |  |  | 3.5 | 0.15 | -5 / 210 | 50 | Flare stack diameter 16"; Flare tip diameter 16" ; Stack ht - 26m (Note 1) | 15.0 | Vendor Specified | API 537 / API 521 |  |  |  |  |
| 430-SK-1708 | CO2 Vent Stack | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0003 | Package Unit | 1814 kg/h (25544 SCMD) |  |  |  | 3.5 | 0.2 | -5 / 150 | 54 | 6" vent Stack ht - 20m | 2.0 | Vendor Specified |  |  |  |  |  |
| 430-EH-1709 | HP Flare KO Drum Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0003 | Electric Heater |  |  | 40 |  | 10.0 |  | -46 / 160 |  |  | 0.5 | Element: 316L SS |  |  |  |  |  |
| 430-EH-1710 | LP Flare KO Drum Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0004 | Electric Heater |  |  | 15 |  | 3.5 |  | -5 / 210 |  |  | 0.3 | Element: 316L SS |  |  |  |  |  |
| NOTE: - HP and LP Flare System capacity and equipment sizes to be confirmed during EPC phase after a Flare and Blowdown Study. - HP and LP Flare Stack packages typically include: - Flare Tip Assembly - Flare Stack and Pilot Gas Riser - Seal - Pilot Gas System - Pilot Gas Back-Up (Propane) - Redundant Ignition and Monitoring Systems required. One of the ignition sources shall be electronic flare ignition type. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 19 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 440

PRODUCED WATER TREATMENT

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | PRODUCED WATER TREATMENT | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 440-V-1301 | Produced Water Flash Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-PFD-0007 | Horizontal Vessel | 10.2 m3/h |  |  |  | FV / 5 | 1.0 | -25 / 85 | 50 | 1.2 ID x 3.5 T/T | 2.8 | CS + 3mm CA ; non-metallic Internal lining | ASME VIII Div I |  |  |  |  |
| 440-P-1302 A/B | Produced Water Feed Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0007 | Centrifugal | 10.2 m3/h @ 3 bar dp | 1.8 | 3.0 |  | 10 | 4 | -5 / 85 | 50 |  | 1.7 | Case: DSS Impeller: DSS API 610 Material Class D1 or D2 | API 610 |  |  | Electric Drive Motor. |  |
| 440-PK-1303 | Produced Water Treatment Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0007 | Note 3 | 10.2 m3/h | 10 |  |  |  |  |  |  |  | 10 | Vendor Specified |  |  |  | HOLD 4 | 002 |
| 440-PIT-1304A/B | Evaporation Pond | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0007 |  |  |  |  |  |  |  | -5 / 85 | 50 | 120 x 120 x 1.5 |  | Geo-Textile Lined |  |  |  | Evaporation Ponds to be internally lined with Geo-Textile Membrane. |  |
| 440-PK-1305A/B | Oil Skimmer Package | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-PFD-0007 |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | One for each Evaporation Pond. HOLD 4. |  |
| 440-PIT-1306 | Offspec Water Pit | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- PFD-0007 |  |  |  |  |  |  |  | -5 / 85 | 50 | 10 x 10 x 1.5 |  | Geo-Textile Lined |  |  |  | To be internally lined with Geo-Textile Membrane |  |
| NOTE: - Produced Water unit capacity based on produced water flow rate of approximately 7 m3/h (Note 2) and a design margin for processing oily water from open drains system and water draw-off from Condensate Stabilisation Column. - Produced Water flow rate of 7 m3/h is pro-rated from the Rich Winter Heat and Material Balance to correspond to 7,000 SBPD condensate product from FACILITY. - The Produced Water Treatment Package typically includes a CPI Separator and an Oil Transfer Pump. Space and tie-ins shall be provided within the package for future installation of secondary treatment, e.g. Induced Gas Flotation (IGF) Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 20 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 450 
FUEL GAS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | FUEL GAS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 450-EH-2401 | Fuel Gas Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0014 | Electrical |  |  | 80 |  | FV / 35 | 30 | -25 / 85 | 30 - 70 |  | 0.8 | CS + 3 mm CA Element: CS |  |  |  |  | 002 |
| 450-KO-2402 | HP Fuel Gas KO Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0014 |  | 0.12 MSCMD |  |  |  | FV / 35 | 30 | -25 / 85 | 30 - 70 | 0.8 ID x 2.4 T/T | 2.5 | CS + 3 mm CA 316L SS internals | ASME VIII Div 1 | Yes |  |  | 002 |
| 450-F-2403A/B | Fuel Gas Filter / Separator | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0014 |  | 0.12 MSCMD |  |  |  | FV / 35 | 30 | -25 / 85 | 30 - 70 | 0.6 x 1.5 | 1.0 | CS + 3 mm CA 316L SS internals | ASME VIII Div 1 | Yes |  |  | 002 |
| 450-KO-2404 | LP Fuel Gas KO Drum | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0014 |  | 0.06 MSCMD |  |  |  | FV / 10 | 5 | -25 / 85 | 30 - 70 | 0.8 ID x 2.4 T/T | 2.0 | CS + 3 mm CA 316L SS internals | ASME VIII Div 1 | Yes |  |  | 002 |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 21 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 460 
METHANOL INJECTION

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | METHANOL INJECTION | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 460-P-2901 | Methanol Offloading Pump | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0006 | Centrifugal | 10 m3/h @ 3 bar dp | 2.8 | 4.0 |  | 7 | 3 | -5 / 85 | 50 |  | 1.0 | Case: CS, Impeller: CS API 610 Material Designation S-4 | API 610 |  |  |  |  |
| 460-TK-2902 | Methanol Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0006 |  | 150 m3 |  |  |  | 0.05 | ATM | -5 / 85 | 50 | 6.0 ID x 7.2 H | 18.0 | CS + 3mm CA with no- metallic internal lining | API 650 |  | Yes | Note 1 |  |
| 460-P-2903A/B | Methanol Distribution Pumps For FACILITY | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0006 | Metering Pump | 0.82 m3/h @ 58 bar dp | 3 | 5.0 |  | 65 | 58 | -5 / 85 | 50 |  | 1.0 | Case: CS, Impeller: CS API 610 Material Designation S-4 | API 675 |  |  | HOLD 3 - Capacity |  |
| 460-P-2904A/B | Methanol Distribution Pumps For Wellheads | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0006 | Diaphragm Pump | 3.1 m3/h @ 320 bar dp | 49 | 60 |  | 383 | 320 | -5 / 85 | 50 |  | 1.0 | Case: CS, Impeller: CS API 610 Material Designation S-4 | API 675 |  |  | Note 1 |  |
| NOTE: - Methanol tank capacity based on one week's capacity considering an ambient soil temperature of 16°C. Wellhead distribution pump capacity based on maximum requirement considering an ambient soil temperature of 10°C. - Methanol supply and unloading requirements to be confirmed in EPC phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 22 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 470 
CHLORINATION

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | CHLORINATION | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 470-PK-2601 | Chlorination Unit | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0015 | Note 1 | Note 2 |  |  |  | 12 | 8 | -5 / 85 | 50 | 2.0 x 2.0 x 3.0 | 2.0 | Titanium / CPVC |  |  |  | Vendor to confirm material selection |  |
| NOTE: - The Chlorination Package comprises chemical tank with mixer and dosing pumps. - Chlorination unit capacity suitable for treatment of 30 m3/h Aquifer Water. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 23 of 35

|  | EQUIPMENT LIST DETAILS SYSTEM CODE: 480 UTILITY MISCELLANEOUS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |  |
|  | Client | COMPANY | Checked By | JL |  |
|  | Location | UTILITY MISCELLANEOUS | Approved By | CF |  |
|  | Job No. | PROJECT NUMBER | Revision | 002 |  |
|  | Doc No. | PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001 | Date | DATE |  |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 480-TK-0801 | Amine Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0001 |  | 29 m3 |  |  |  | 0.05 | ATM | -5 / 85 | 50 | 3.35 m OD x 4.6 m H | 4.5 | CS • 3mm CA with non- metallic internal lining. System 5 up to 1m from tank bottom | API 12F |  |  |  |  |
| 480-A-0802 | Amine Storage Tank Mixer | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0001 |  |  |  | 5.5 |  |  |  |  |  |  | 0.2 | 316L SS |  |  |  |  |  |
| 480-P-0803A/B | Amine Transfer Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0001 | Centrifugal | 5 m3/h @ 6 bar dp | 2.1 | 3.0 |  | 10 | 6 | -5 / 85 | 50 |  | 1.0 | Case: 316L SS, Impeller: 316L SS API 610 material designation S-9 | API 610 |  |  |  |  |
| 480-EH-0804 | Amine Storage Tank Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0001 | Electric |  |  | 10.0 |  |  |  |  |  |  |  | Element: 316L SS |  |  |  |  |  |
| 480-V-0805 | Amine Drain Vessel | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0001 | Horizontal Vessel | 15 m3 |  |  |  | FV / 3.5 | 0.01 | -5 / 85 | 50 | 2.2 x 6.0 (ID x TT) | 5.5 | CS • 3 mm CA External lining and CP | ASME VIII Div 1 |  |  |  |  |
| 480-P-0806A/B | Amine Drain Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0001 | Sump Pump | 5 m3/h @ 5 bar dp | 1.7 | 3.0 |  | 10 | 5 | -5 / 85 | 50 |  | 0.7 | Case: 316L SS, Impeller: 316L SS API 610 material designation S-9 | API 610 |  |  |  |  |
| 480-EH-0807 | Amine Drain Vessel Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0001 | Electric |  |  | 5.0 |  | 3.5 |  |  |  |  | 0.2 | Element: 316L SS |  |  |  |  |  |
| 480-TK-0808 | Glycol Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0002 |  | 15 m3 |  |  |  | 0.05 | ATM | -5 / 85 | 50 | 2.93 OD x 3.65 H | 3.5 | CS • 3mm CA | API 12F |  |  |  |  |
| 480-P-0809A/B | Glycol Transfer Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0002 | Centrifugal | 3 m3/h @ 6 bar dp | 1.2 | 3.0 |  | 11 | 6 | -5 / 85 | 50 |  | 1 | Case: 316L SS, Impeller: 316L SS API 610 material designation S-9 | API 610 |  |  |  |  |
| 480-EH-0810 | Glycol Storage Tank Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0002 | Electric |  |  | 10.0 |  |  |  |  |  |  | 0.2 | Element: 316L SS |  |  |  |  |  |
| 480-V-0811 | Glycol Drain Vessel | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0002 | Horizontal Vessel | 10 m3 |  |  |  | FV / 3.5 | 0.01 | -5 / 85 | 50 | 1.9 x 6.0 (ID x TT) | 4.5 | CS • 3 mm CA External lining and CP | ASME VIII Div 1 |  |  |  |  |
| 480-P-0812A/B | Glycol Drain Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0002 | Centrifugal | 5 m3/h @ 5 bar dp | 1.7 | 3.0 |  | 11 | 5 | -5 / 85 | 50 |  | 1.00 | Case: 316L SS, Impeller: 316L SS API 610 material designation S-9 | API 610 |  |  |  |  |
| 480-EH-0813 | Glycol Drain Vessel Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0002 | Electric |  |  | 5.0 |  | 3.5 |  |  |  |  | 0.20 | Element: 316L SS |  |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 24 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 500

AQUIFER / PLANT WATER

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | AQUIFER / PLANT WATER | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 500-F-2601A/B | Aquifer Water Filter | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 |  | 30 m3/h |  |  |  | 12 | 8 | -5 / 85 | 50 |  | 0.5 | CS + Rubber lining | ASME VIII Div 1 |  |  |  |  |
| 500-TK-2602A/B | Plant Water Storage Tank | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0015 | Note 1 | 115 m3 |  |  |  | ATM | ATM | -5 / 85 | 50 | 4.7 OD x 7.3 H | 10 | CS + 3mm CA with non-metallic lining | API 12F |  |  |  |  |
| 500-P-2603A/B | Plant Water Transfer Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 |  | 32 m3/h @ 8.5 bar dp | 15.0 | 20.0 |  | 12 | 8 | -5 / 85 | 50 |  | 1.0 | Case: 316L SS, Impeller: 316L SS | ISO 5199 / ASME B73.1 |  |  |  |  |
| NOTE: 1. Plant water capacity quoted is nominal capacity corresponding to two days storage of normal requirement of plant water (includes requirements for production of fresh water and demin water) and make-up for fire water. See also Note 1 on Page 26. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 25 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 530

FRESH/POTABLE/DEMINERALISED WATER

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | FRESH/POTABLE/DEMINERALISED WATER | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 530-PK-3101 | Fresh Water RO Treatment Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 |  | 1.75 m3/h |  | 5.0 |  | 8 | 5.0 | -5 / 85 | 50 |  | 5.0 | 316L SS (Vendor Specified) |  |  |  | HOLD 4 |  |
| 530-TK-3102A/B | Fresh Water Storage Tank | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0015 |  | 50 m3 each |  |  |  | ATM | ATM | -5 / 85 | 50 | 3.66 OD x 6.1 H | 7.0 | CS + 3mm CA with non-metallic lining | API 12F |  |  |  |  |
| 530-P-3103A/B | Fresh Water Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 |  | 35 m3/h @ 5.5 bar dp | 10.5 | 15.0 |  | 8 | 5.7 | -5 / 85 | 50 |  | 1.0 | Case: 316L SS, Impeller: 316L SS | ISO 5199 / ASME B73.1 |  |  |  |  |
| 530-PK-3104 | Demin Water Polishing Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0015 |  | 0.5 m3/h |  | 5.0 |  | 8 | 3 | -5 / 85 | 50 |  | 1.0 | Vendor specified | ISO 5199 / ASME B73.1 |  |  | HOLD 4 |  |
| 530-TK-3105 | Demin Water Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0015 |  | 45 m3 |  |  |  | 0.05 | ATM | -5 / 85 | 50 | 3.66 OD x 6.1 H | 7.0 | CS + 3mm CA with non-metallic lining | API 12F |  |  |  |  |
| 530-P-3106A/B | Demin Water Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 |  | 1.6 m3/h @ 4.7 bar dp | 1.0 | 2.2 |  | 8 | 4.7 | -5 / 85 | 50 |  | 1.0 | Case: 316L SS, Impeller: 316L SS | ISO 5199 / ASME B73.1 |  |  |  |  |
| 530-PK-3107 | Chemical Dosing Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0015 | Note 3 |  |  |  |  | 8 | 5 | -5 / 85 | 50 |  | 12.0 | 316L SS (Vendor Specified) | API 675 |  |  | Note 2 |  |
| NOTE: - Fresh water and demin water storage capacity quoted is required working volume corresponding to seven days storage of their normal requirements. - Chemicals for water treatment typically are biocide and regeneration chemicals for Demineralisation Unit. - Chemical Dosing Package comprises (HOLD 4): - Multi-compartmented Tank (3 compartments) - Dosing Pump (3 operating + 3 standby) - Mixers (3; one per compartment) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 26 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 560 
OPEN DRAINS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | OPEN DRAINS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 560-PIT-3001 | Open Drains Sump | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0005 |  | NOTE 1 |  |  |  | ATM | ATM | -5 / 85 | 50 | 4.0 x 4.0 x 2.5 |  | Lined Concrete |  |  |  |  |  |
| 560-P-3002A/B | Open Drains Sump Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0005 |  | 2 m3/h @ 3.0 bar dp | 0.5 | 1.1 |  | 8.0 | 3.0 | -5 / 85 | 50 |  | 1.0 | Case: UNS S31803 Impeller: UNS S3183 API 610 Material Designation D1 |  |  |  |  |  |
| 560-A-3003 | Open Drain Oil Skimmer Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0005 |  |  |  |  |  |  |  |  |  |  |  | Vendor Specified |  |  |  | Located within Open Drains Sump |  |
| NOTE: 1. The Open Drains Sump is designed to handle a maximum drainage volume of 5 m3 in one day at a maximum rate of 15 m3/h. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 27 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 570 
CLOSED DRAINS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | CLOSED DRAINS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 570-V-2101 | Closed Drains Vessel | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0005 |  | 32 m3 |  |  |  | 3.5 | 0.1 | -5 / 85 | 50 | 2.7 ID x 8.5 T/T | 8.7 | Shell: CS + 3mm CA with non-metallic lining, external coating and CP | ASME VIII Div 1 | Internal Lining (Epoxy Coating) Located in a Concrete Pit |  |  |  |
| 570-P-2102A/B | Closed Drains Transfer Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0005 | Centrifugal | 5 m3/h @ 22 bar dp | 10.0 | 14.0 |  | 31 | 22.0 | -5 / 85 | 50 |  | 0.6 | Case: UNS S31803 Impeller: UNS S3183 API 610 Material Designation D1 | API 610 |  |  |  |  |
| 570-EH-2103 | Closed Drains Vessel Heater | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0005 | Electric |  |  | 40 |  | 3.5 |  | -5 / 85 |  |  |  | Element: 316L SS |  |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 28 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 620 
DIESEL OIL

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | DIESEL OIL | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 620-TK-2001 | Diesel Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0011 |  | 60 m3 |  |  |  | 0.05 | ATM | -5 / 85 | 50 | 4.7 OD x 4.9 H | 9.0 | CS + 3mm CA with non-metallic internal lining at bottom of tank | API 12F |  |  | Capacity based on 3 days storage at maximum diesel consumption. |  |
| 620-P-2002A/B | Diesel Transfer Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0011 |  | 5 m3/h @ 2.5 bar dp | 1.0 | 2.2 |  | 7 | 4.0 | -5 / 85 | 50 |  | 1.0 | Case: CS Impeller: Cast iron API 610 material designation S-1 | API 610 |  |  |  |  |
| 620-FC-2003 | Diesel Filter | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0011 |  | 5 m3/h |  |  |  | 7 | 4.0 | -5 / 85 | 50 |  | 1.0 | CS + 3mm CA with non-metallic internal lining |  |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 29 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 630

INSTRUMENT AND PLANT AIR

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | INSTRUMENT AND PLANT AIR | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. |  | °CMaterials Size L x W x H m | Weight (each) tonnes |  | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 630-K-2201A/B | Air Compressor | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0012 | Oil-free Screw type | 760 Sm3/h | 111 | 132 |  | 12 | 10.0 | -5 / 85 | 50 |  | 4 | CS (Vendor specified) | API 619 / ISO10440 |  |  | Note 1, HOLD 4 | 002 |
| 630-VL-2202 | Plant Air Receiver | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0012 |  | 15 m3 |  |  |  | 12 | 10.0 | -5 / 85 | 50 | 1.8 ID x 5.4 T/T | 6 | Shell: CS + 3mm CA with non-metallic internal lining | ASME VIII Div 1 |  |  |  | 002 |
| 630-PK-2203A/B | Air Drier Package | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0012 | Each comprises two towers. | 660 Sm3/h |  |  |  | 12 | 9.5 | -5 / 85 | 50 |  | 5 | Vendor Specified | ASME VIII Div 1 |  |  | Capacity quoted is instrument (dry) air flow rate. Note 1, HOLD 4 | 002 |
| 630-VL-2204 | Instrument Air Receiver | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0012 |  | 40 m3 |  |  |  | 12 | 9.0 | -5 / 85 | 50 | 2.5 ID x 7.3 T/T | 11 | Shell: CS + 3mm CA with non-metallic internal lining | ASME VIII Div 1 |  |  |  | 002 |
| NOTE: 1. The maximum instrument air demand is 660 Sm3/hr. The Air Compressor Capacity includes an additional 15% margin which has been assumed for drier bed regeneration, giving a total air compressor capacity of 760 Sm3/hr. Supplier to confirm the regeneration air flow requirement. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 30 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 640 
INERT GAS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | INERT GAS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 640-PK-2701 | Nitrogen Generator Package | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0013 | Note 1 | 90 Sm3/h |  | 5 |  | 12 | 9 | -5 / 85 | 50 |  | 1.0 | Vendor specified |  |  |  | HOLD 4 | 002 |
| 640-RCV-2702 | Nitrogen Receiver | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0013 | Vertical Process Vessel | 38 m3 |  |  |  | 12 | 7 | -5 / 85 | 50 | 2.5 x 7.0 (ID x TT) | 11.0 | Shell: CS + 3mm CA with 316L SS internals | ASME VIII DIV I |  |  |  | 002 |
| 640-PK-2703 | High Purity Nitrogen Bottles Skid | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0013 |  | 20 Sm3/h |  |  |  |  |  |  |  |  |  | Vendor Specified |  |  |  | HOLD 4 |  |
| NOTE: 1. Nitrogen Generation Package typically includes: - Inlet Carbon Filters (2x100%) - Inlet Heater (1x100%) - Membrane Separators (with 20% spare membranes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 31 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 660 
SEWAGE TREATMENT

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | SEWAGE TREATMENT | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure |  | barg(each) Temperature. °C |  | Size L x W x H m | Weight tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 660-PK-3201 A/B/C/D | Sewage Sump Package | 4 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0016 | Septic Tank | 5.0 m3 |  |  |  | Vendor Specified | Vendor Specified | -5 / 85 | 50 |  | 1.0 | Vendor Specified |  |  |  | HOLD 3 - quantity, capacity | 002 |
| NOTE: 1. Equipment Tag 660-PK-3202 (Waste Water Treatment Package) has been deleted in Rev 002 of this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 002 |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 32 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 710 
FIRE WATER

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | FIRE WATER | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 710-TK-2301 | Firewater Storage Tank | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0007 | Note 1 | 2836 m3 |  |  |  | ATM | ATM | -5 / 85 | 50 | 18 ID x 14.4 H | 120 | CS + 3mm CA with non-metallic internal lining | API 650 |  |  |  | 002 |
| 710-P-2302 | Electric Firewater Pump | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 |  | 350 m3/h ® 10 bar dp | 130 | 150 |  | 16 | 10.0 | -5 / 85 | 50 |  | 6 | Case: Ni Al Bz Impeller: Ni Al Bz | NFPA 20 |  |  | 3 x 50% Firewater pump configuration | 002 |
| 710-PK-2303A/B | Diesel Firewater Pump Package | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 | Note 4 | 350 m3/h ® 10 bar dp | 130 | 150 |  | 16 | 10.0 | -5 / 85 | 50 |  | 15.0 | Vendor Specified | NFPA 20 |  |  | 3 x 50% Firewater pump configuration | 002 |
| 710-P-2305A/B | Fire Water Jockey Pump | 2 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 |  | 10 m3/h ® 10 bar dp | 8.0 | 11 |  | 16 | 10.0 | -5 / 85 | 50 |  | 2.0 | Case: Ni Al Bz Impeller: Ni Al Bz | NFPA 20 |  |  | 2 x 100% electric pumps | 002 |
| 710-A-2306 | Foam Package (Condensate Storage Area) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 | Note 2 | 21 m3/h ® 11 bar dp | 11 | 15 |  | 16 | 11.0 | -5 / 85 | 50 |  | 2.0 | CS + 3mm CA with non-metallic internal lining | NFPA 20 API 650 |  |  | HOLD 4 |  |
| 710-A-2307 | Foam Package (Methanol Storage Area) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 | Note 3 |  |  |  |  |  |  |  |  |  |  | CS + 3mm CA with non-metallic internal lining | NFPA 20 API 650 |  |  | HOLD 4 |  |
| 710-A-2310 | Deluge Skid (Methanol Tank) (460-TK-2902) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0007 |  |  |  |  |  |  |  |  |  |  |  |  | NFPA 15 |  |  | HOLD 4 |  |
| 710-A-2311 | Deluge Skid (Condensate Tank) (330-TK-1501) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0007 |  |  |  |  |  |  |  |  |  |  |  |  | NFPA 15 |  |  | HOLD 4 |  |
| 710-A-2312 | Deluge Skid (Condensate Tank) (330-TK-1502) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR-UFD-0007 |  |  |  |  |  |  |  |  |  |  |  |  | NFPA 15 |  |  | HOLD 4 |  |
| 710-A-2313 | Deluge Skid (Offspec Condensate Tank) (330-TK-1503) | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0007 |  |  |  |  |  |  |  |  |  |  |  |  | NFPA 15 |  |  | HOLD 4 |  |
| NOTE: - Fire Water Unit designed to cater for only the Condensate Storage Tanks. Accordingly, there will be no ring main and jockey pumps - Foam Package (Condensate Storage Area) contains Foam Pump and Foam Storage Tank (20m3). - Foam Package (Methanol Storage Area) contains Foam Storage Tank (1.35 m3). - Diesel Firewater Pump Package also contain diesel tank storage of 1 m3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 33 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 800

MAIN POWER GENERATION

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | MAIN POWER GENERATION | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 800-GT-2001 | Gas Turbine Generator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0010 | Note 1 |  | 4.0 MW |  |  |  |  |  |  | 12 x 5.8 x 8.1 | 60 | Vendor Standard | API STD 616 |  |  | Typically 6.75 MW ISO rated gas turbine (SGT-200) |  |
| 800-GT-2002 | Gas Turbine Generator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0010 | Note 1 |  | 4.0 MW |  |  |  |  |  |  |  | 60 | Vendor Standard | API STD 616 |  |  | Typically 6.75 MW ISO rated gas turbine (SGT-200) |  |
| NOTE: - 2 x 100% duty Gas Turbine Generator Packages, each for 4 MW power output. - Overall package dimensions for 2 machines. GTG skid alone is significantly smaller. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 34 of 35

EQUIPMENT LIST DETAILS 
SYSTEM CODE: 830

EMERGENCY POWER GENERATORS

| Project | PROJECT1 DEVELOPMENT PROJECT | Originated By | LY |
| --- | --- | --- | --- |
| Client | COMPANY | Checked By | JL |
| Location | EMERGENCY POWER GENERATORS | Approved By | CF |
| Job No. | PROJECT NUMBER | Revision | 002 |
| Doc No. | Project1-FEED CONTRACTOR--MUL-E000-PR-LST-0001 | Date | DATE |

| Tag Number | Equipment Name | QTY | PFD No. | Package Notes | Capacity | Duty kW | Instal. Power kW | Surface Area (m2) | Pressure barg |  | Temperature. °C |  | Size L x W x H m | Weight (each) tonnes | Materials | Design Code | Internals | Insulation | Remarks | Rev Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Design | Oper | Design | Oper |  |  |  |  |  |  |  |  |
| 830-GD-2001 | Emergency / Standby Diesel Generator | 1 | PROJECT1-FEED CONTRACTOR-PF01-E000-PR- UFD-0010 |  |  | 750 |  |  |  |  |  |  | 7.5 x 4.0 x4.0 | 30 | CS (Vendor Standard) | IEC-60034 |  |  |  |  |
| NOTE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

PROJECT1-FEED CONTRACTOR-MUL-E000-PR-LST-0001_Rev002 -	Page 35 of 35