How to change spark

plugs on

TOYOTA

COROLLA (ZZE12_,

NDE12_, ZDE12_)

–

replacement guide

SIMILAR VIDEO TUTORIAL

This video shows the replacement procedure of a similar car part on another vehicle

This replacement procedure can be used for:

TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_) 1.8 VVTL-i TS (ZZE123_), TOYOTA

COROLLA (ZZE12_, NDE12_, ZDE12_) 1.8 VVTL-i TS (ZZE123)

The steps may slightly vary depending on the car design.

Important!

This tutorial was created based on the replacement procedure for a similar car part on: TOYOTA RAV 4 III (ACA3_, ACE_, ALA3_, GSA3_, ZSA3_) 2.4 (ACA33)

Buy tool

s

REPLACEMENT: SPARK PLUGS – TOYOTA COROLLA

(

ZZE12_, NDE12_, ZDE12_). TOOLS YOU MIGHT NEED

:

Electronic spray

High-temperature anti-seize lubricant

Drive socket # 10

Spark plug socket # 16

Ratchet wrench

Torque wrench

Fender cover

The replacement procedure is the same for all spark plugs.

All work should be done with the engine stopped.

Replacement: spark plugs – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_).

Tip from AUTODOC:

REPLACEMENT: SPARK PLUGS – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_). TAKE THE FOLLOWING STEPS:

1

Open the hood.

2

Use a fender protection cover to prevent damaging paintwork and plastic parts of the car.

3

Unscrew the engine cover fastener. Use a drive socket #10. Use a ratchet wrench.

4

Remove the engine cover.

5

Unplug the connectors from the ignition coils.

6

Unscrew the ignition coil bracket. Use a drive socket #10.

7

Remove the fastening bolts.

8

Take out the ignition coil from the spark plug well.

Remember the sequence of steps for ignition coil demounting.

Replacement: spark plugs – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_).

Tip:

9	Clean dirt, dust and debris from inside the spark plug wells.

10

11

Take out the spark plugs.

12

Unscrew the spark plugs. Use the spark plug socket #16. Use a ratchet wrench.

Apply a thin layer of a special high temperature anti-seize lubricant to the threads of the new spark plugs. Treat the surface of the terminals of the new spark plugs. Use high-temperature grease.

13	Install the new spark plugs into their mounting seats.

14

Screw new spark plugs and tighten them. Use the spark plug socket #16. Use a torque wrench. Tighten it to 25

nm torque.

15

Install the ignition coils to the spark plug wells. Make sure you hear a click indicating that it is locked in place.

Don't tighten the plugs with excessive force. Exceeding tightening torque can

damage the thread connection.

Be careful when working with a spark plug wrench. Its skewing can cause

damage to the threaded connection.

Replacement: spark plugs – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_).

Tip from AUTODOC experts:

16

Install the fastening bolts.

17

Tighten the fasteners of the ignition coils. Use a drive socket #10. Use a torque wrench. Tighten it to 8 nm torque.

18

Treat the ignition coil connector with electronic spray. The spray will protect the electrical contacts from

corrosion, oxidation, and will reduce the contact resistance.

Replacement: spark plugs – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_).

Follow the installation steps.

AUTODOC recommends:

19

Plug in the connectors of the ignition coils.

Make sure you hear a click indicating that it is locked in place.

20

Install the engine cover and secure it.

21

Screw in the engine cover fasteners. Use a drive socket #10. Use a ratchet wrench.

Make sure the engine cover is properly seated. It should not be displaced with

respect to the guiding fasteners.

Replacement: spark plugs – TOYOTA COROLLA (ZZE12_, NDE12_, ZDE12_). Do

not use excessive force during installation. This may damage the fasteners.

AUTODOC recommends:

22

23

24

25

Run the engine for a few minutes. This is necessary in order to make sure that the component operates properly.

Shut off the engine.

Remove the fender protection cover.

Close the hood.

VIEW MORE TUTORIAL

S

WELL DONE!

AUTODOC — TOP QUALITY AND AFFORDABLE CAR

# PARTS ONLINE

AUTODOC MOBILE APP: GREAT DEALS AND CONVENIEN

T

SHOPPIN

G

A GREAT SELECTION OF SPARE PARTS FOR YOUR CA

R

SPARK PLUGS: A WIDE SELECTIO

N

DISCLAIMER:

The document contains only general recommendations that may be useful for you when you perform repair or replacement work. AUTODOC shall not be liable for any loss, injury, damage of property occurring in the repair or replacement process due to incorrect use or misinterpretation of the provided information.

AUTODOC shall not be liable for any possible mistakes and uncertainties in this guide. The information provided is for information purposes only and cannot replace advice from specialists.

AUTODOC shall not be liable for incorrect or hazardous usage of equipment, tools and car parts. AUTODOC strongly recommends to be careful and observe the safety rules when performing repair or replacement works. Remember: usage of low quality auto parts does not guarantee you the appropriate level of road safety.

© Copyright 2022 – All the contents of this website, in particular texts, photographs and graphics, are protected by copyright. All rights, including reproduction, publication, editing and translation rights, are reserved by AUTODOC GmbH.